[2 marks]

- A hexadecimal number identifies a sequence of bits: 6B16
Translate the value to decimal, assuming...

- 6B16 = 0110 10112

- a) an 8-bit word, 2's comp

- 0110 10112 = 26 + 25 + 23 + 21 + 20 = 64 + 32 + 8 + 2 + 1 = 10710

- b) an 8-bit word, 2's comp., 3-bit prec.

- 0110 1.0112 = 23 + 22 + 20 + 2-2 + 2-3 = 23 + 22 + 20 + 1/(22) + 1(23) = 8 + 4 + 1 + 0.25 + 0.125 = 13.37510

- or   10710 / 23 (because of 3-bit prec.) = 107 / 8 = 13.37510

[1 mark]

- Show the largest positive and largest negative values in fixed-point: 8-bit word, 2's comp, 3-bit prec?   Answer in decimal.

- largest positive:	0111 1.1112 = 15.87510

- largest negative:	1000 0.0002 = -16.010

[1 mark]

- Which part of floating-point (FP) is associated with "overflow errors," and which part with "underflow" errors?

- overflow errors indicate an error in magnitude, and are associated with the exponent.

- underflow errors indicate an error in precision, and are associated with the mantissa (significand).

[2 marks]

- Identify the valid FP representations.  Assume a format: 6-bit exponent, 9-bit mantissa.

- s    exp             mantissa

- a)  1 0 0 0 1 0 1 0 0 0 1 1 1 0 0 0			valid? _NO_

- b)  0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 1			valid? _YES_

- c)  0 1 0 0 0 0 0 1 0 0 0 0 0 0 0 0			valid? _YES_

- d)  1 0 1 1 1 1 1 0 1 1 1 1 1 1 1 1			valid? _NO_

[2 marks]

- Show the largest and smallest values of the FP format: 6-bit exponent, 9-bit mantissa
Do not translate to decimal.

- largest:

- x  0 1 1 1 1 1  1 1 1 1 1 1 1 1 1	most positive exponent, mantissa largest values (all 1's)

- smallest:

- x  1 0 0 0 0 0  1 0 0 0 0 0 0 0 0	most negative exponent, mantissa smallest value conforming to the ".1" rule

** remember that the sign bit in the representation does not play a role in range (magnitude, precision)

- [3 marks]

- Assume the FP format is as above.  What value is represented by the 16-bit word?
Answer in decimal

0 0 0 0 1 0 1 1 0 0 1 1 1 0 0 0

1. sign bit: 0 – true value is positive

2. exponent: 0001012 = 510

3. mantissa: .100111000 => normalised value being stored:  .1001112 * 25

4. true value => 10011.12 = 24 + 21 + 20 + 2-1 = 16 + 2 + 1 + .5 = 19.510