THE UNIVERSITY COLLEGE

OF THE CARIBOO

COMPUTING 253

Small Computer Systems: Organisation and Architecture

Take Home Review – Numeric Representation
"Fixed and Floating—What's the Point?"

For completeness, show your work wherever possible.

- Using the direct method of calculating the decimal form of any 2's complement number (whether the number is positive or negative).

- Direct method:

- let the msb (sign bit) have a data value of  –(2(n-1))10 ; n is the integer word size (to the left of point)

- add the unit values of the remaining bits to the first number (which is either zero, or a large negative)

- since the msb unit value is larger than all other bits put together, if the msb = 1 the outcome is negative, if msb = 0 the outcome is positive

- note: this method does perform any strange "flip & add 1" techniques

- Use this technique to determine the values of the binary fixed-point numbers below,

- using 8-bit word, 2’s complement,

a) 0000 00112 = ?10 	b) 1000 00112 = ?10

- using 8-bit word, 2’s complement, 3-bit precision,

c) 00010.1012 = ?10 	d) 10010.1012 = ?10

- Convert and calculate the following with fixed-point on an 8-bit word and 4-bit precision (all-positive),

a) What is the largest value that can be stored?  (Answer in binary and decimal.)

b) 10.5010 = ?2 		c) 6.062510 = ?2

d) 1010.10102 = ?10 	e) 8216 = ?10

- Convert and calculate the following in fixed-point form with: 8-bit word, 2's complement, and 3-bit precision,

a) What are the largest positive and largest negative values?  (Answer in binary and decimal.)

b) -10.5010 = ?2 		c) 13.37510 = ?2 		d) -13.37510 = ?2

e) 10101.0102 = ?10 	f) A216 = ?10

f) in binary, calculate the result of the value in c) – the value in e)

- Express the following decimal values in floating-point form with: 16-bit word, 7-bit exponent, and 8-bit mantissa,

a) 0.010    b) 1.010    c) –0.510    d) –5.6210	e) 1/64 10

- Express the following floating-point numbers in decimal ( __ 10 ).  (use floating-point structure as in question 4.)

a) 0|000 0000|1000 0000

b) 0|000 0010|1111 0000

c) 1|111 1111|1010 0000

d) 1|000 0000|1001 0100

e) 0|111 1111|1100 0000

f) What is the largest number and smallest number that can be stored in this FP format?

- The following floating-point numbers are invalid, and consider NaN ("not-a-number") in floating-point representation.  Indicate why.

a) 0|000 0010|0101 0100

b) 1|000 0000|0000 0000

c) 0|000 0100|0000 0100

d) 1|100 0000|0000 0001