THE UNIVERSITY COLLEGE

OF THE CARIBOO

COMPUTING 253

Small Computer Systems: Organisation and Architecture

Take Home Review – Numeric Representation
"Fixed and Floating—What's the Point?"

- Using the "true meaning of 2's complement" method,

- using 8-bit word, 2’s complement,

a)  0000 00112 = ?10

= -(27)*0 + 21 + 20 = -128*0 + 2 + 1 = 310

b)  1000 00112 = ?10

= -(27)*1 + 21 + 20 = -128*1 + 2 + 1 = -12510

- using 8-bit word, 2’s complement, 3-bit precision,

c)  00010.1012 = ?10

= -(24)*0 + 21 + 2-1 + 2-3 = -16*0 + 2 + 0.5 + 0.125 = 2.62510

d)  10010.1012 = ?10

= -(24)*1 + 21 + 2-1 + 2-2 = -16*1 + 2 + 0.5 + 0.125 = -13.37510

- Convert and calculate the following with fixed-point on an 8-bit word and 4-bit precision (all-positive),

a)  What is the largest number that can be stored?  (Answer in binary and decimal.)

in binary: 1111.11112

in decimal: 23+22+21+20 + 2-1+2-2+2-3+2-4

= 8+4+2+1+0.5+0.25+0.125+0.0625 = 15.937510

b)  10.5010 = ?2

A) 10 / 2 = 5 + rem 0 : 0 (ls)

5 / 2 = 2 + rem 1  : 1

2 / 2 = 1 + rem 0  : 0

1 / 2 = 0 + rem 1  : 1 (ms)   : 1010 = 10102

B) .50 * 2 = 1.0 : 1 (ms)/(ls)

.0 * 2 = 0.0                  : 0.510 = .12

C)  10.5010 = 1010.10002

c)  6.062510 = ?2

A) 6 / 2 = 3 + rem 0 : 0 (ls)

3 / 2 = 1 + rem 1 : 1

1 / 2 = 0 + rem 1 : 1 (ms)    : 710 = 01102

B) .0625 * 2 = 0.125 : 0 (ms)

.125 * 2 = 0.25   : 0

.25 * 2 = .50     : 0

.50 * 2 = 1.0     : 1 (ls)

.0 * 2 -= 0.0                 : 0.062510 = .00012

C)  7.062510 = 0110.00012

d)  1010.10102 = ?10

(straight): 23+21 + 2-1+2-3 = 8+2+.50+.125 = 10.62510

f)  8216 = ?10

keeping in mind that hexadecimal is a short-cut/compressed form of binary,

=    8   216

=  1000 0010 => 1000.00102  (based on 8-bit word, 4-bit prec, all pos.)

1000.00102 = 23 + 2-3 = 8+.125 = 8.12510

- Convert and calculate the following with fixed-point on an 8-bit word, 2's complement, and 3-bit precision,

a)  What are the largest positive and largest negative numbers?  (Answer in binary and decimal.)

largest positive:

in binary: 01111.1112

in decimal: 23+22+21+20 + 2-1+2-2+2-3

= 8+4+2+1+0.5+0.25+0.125 = 15.87510

largest negative:

in binary: 10000.0002  (by rule of numeric cycle; largest positive + .001)

in decimal: (using method in #9) -(24) = -16.010

b)  -10.5010 = ?2

(from above: 10.5010 = 01010.1002)

-10.5010 : 1. flip:  10101.011

2. add 1:+     .001

10101.1002

c)  13.37510 = ?2

A) 13 / 2 = 6 + rem 1 : 1 (ls)

6 / 2 = 3 + rem 0  : 0

3 / 2 = 1 + rem 1  : 1

1 / 2 = 0 + rem 1  : 1 (ms)   : 1310 = 011012

B) .375 * 2 = 0.75 : 0 (ms)

.75 * 2 = 1.5   : 1

.50 * 2 = 1.0   : 1 (ls)

.0 * 2 = 0.0                  : 0.37510 = .0112

C)  13.37510 = 01101.0112

d)  -13.37510 = ?2

(from previous: 13.37510 = 01101.0112)

-13.37510 : 1. flip:  10010.100

2. add 1:+     .001

10010.1012

e)  10101.0102 = ?10

(straight): -(24)+22+20 + 2-2 = -16+4+1+.25 = -10.7510

(long method): 1. flip:  01010.101

2. add 1:+     .001

01010.1102 = 23+21 + 2-1+2-3 = 8+2+.5+.25 = 10.7510

and the number is originally negative, so = -10.7510

f)  A216 = ?10

keeping in mind that hexadecimal is a short-cut/compressed form (representation) of binary,

=    A   216

=  1010 0010 => 10100.0102  (based 8-bit word, 3-bit precision)

10100.0102 = -(24)+22 + 2-2 = -16+4 +.25 = -11.7510

g)  in binary, calculate the result of the value in c) – the value in e)

13.37510 => 01101.011 - 10101.0102 = ?2

Knowing the subtraction is performed by "adding a negative," the second value must be negated, yet this number is already negative  this implies that the second value becomes positive.

Therefore,

01101.011 - 10101.0102 = 01101.011 + (-10101.010)2

= 01101.011 +   01010.110 2

= 11000.0012

The result is a negative value instead of a positive because of overflow.

- Express the following decimal values in floating-point form with a 16-bit word, 7-bit exponent, and 8-bit mantissa,

a)  0.010

A) 0.010 = 0.02

B) normalise: 0.02 = 0.02 * 20  (exponent = 010)

C) exponent: 010 = 02

D) sign bit = 0 (positive), store: 0 0000000 00000000

or just conclude: no sign bit, no exponent, no mantissa = 0 0000000 00000000

(zero is a special floating-point value that is commonly just stored as: 0)

b)  1.010

A) 1.010 = 1.02

B) normalise: 1.02 = 0.12 * 21  (exponent = 110)

C) exponent: 110 = 00000012

D) sign bit = 0 (positive), store: 0 0000001 10000000

c)  -0.510

A) 0.510 = 0.12

B) normalise: 0.12 = 0.12 * 20  (exponent = 010)

C) exponent: 010 = 00000002

D) sign bit = 1 (negative), store: 1 0000000 10000000

d)  -5.6210

A) 5.6210 = 101.10011111012  (10 bits precision is good enough!)

B) normalise: 101.10011111012 = .10110011111012 * 23  (exponent = 310)

C) exponent: 310 = 00000112

D) sign bit = 1 (negative), store: 1 0000011 10110011

(the stored floating-point value suffers from underflow.)

e)  1/64 10

A) 1/64 = 2-6  = 0.0000012

B) normalise: 0.0000012 = 0.12 * 2-5  (exponent = -510)

C) exponent: -510 = (510 = 00001012; -510 = 11110112) = 1111011

D) sign bit = 0 (positive), store: 0 1111011 10000000

- Express the following floating-point numbers in decimal ( __ 10 ), 16-bit word, 7-bit exponent, and 8-bit mantissa,

a) 0|000 0000|1000 0000

A) sign bit: 0 - positive

B) exponent: 000 00002 = 010

C) mantissa: .12 * 20 = .12

D) decimal : (2-1) = 0.510

b) 0|000 0010|1111 0000

A) sign bit: 0 - positive

B) exponent: 000 00102 = 210

C) mantissa: .11112 * 22 = 11.112

D) decimal : (21+20+2-1+2-2) = 3.7510

c) 1|111 1111|1010 0000

A) sign bit: 1 - negative

B) exponent: 111 11112 = -110

C) mantissa: .1012 * 2-1 = .01012

D) decimal : -(2-2+2-4) = -0.312510

d) 1|000 0000|1001 0100

A) sign bit: 1 - negative

B) exponent: 000 00002 = 010

C) mantissa: .1001012 * 20 = .1001012

D) decimal : -(2-1+2-4+2-6) = -0.57812510

e) 0|111 1111|1100 0000

A) sign bit: 0 - positive

B) exponent: 111 11112 = -110

C) mantissa: .112 * 2-1 = .0112

D) decimal : (2-2+2-3) = 0.37510

f) What is the largest number and smallest number that can be stored in this FP format?

largest

A) sign bit: 0 or 1 (irrelevant when referring only to magnitude or prec.)

B) exponent: 011 11112 = 6310

C) mantissa: .111111112 * 263

D) decimal : left for discussion

smallest

A) sign bit: 0 or 1 (irrelevant when referring only to magnitude or prec.)

B) exponent: 100 00002 = -6410

C) mantissa: .12 * 2-64

D) decimal : left for discussion

- The following floating-point numbers are invalid.  Indicate why.

a) 0|000 0010|0101 0100

- mantissa does not begin with 0.1; indicating the mantissa was not normalised correctly, or an error occurred in storing the bits

b) 1|000 0000|0000 0000

- exponent and mantissa describe zero, but sign bit indicates a negative; negative zero is not a valid FP number.

c) 0|000 0100|0000 0100

- same problem as a), mantissa is invalid.

d) 1|100 0000|0000 0001

- same problem as a) and c), mantissa is invalid
(note: this FP number does not represent the smallest number; although the exponent is the most negative value possible exponent and the mantissa seems the smallest value, the mantissa is not normalised and the FP is invalid)