COMP253 — Lecture 9: Question C Solutions

Problem:

Derive the final expressions for both Z1 and Z2,

“A circuit that takes in 4 inputs and turns on one circuit (Z1) if the total number of inputs is less than, or equal, to 2 (0,1,2), and another circuit (Z2) if the total number of inputs is 3 or 4."

Solution:

Logic table of circuit:			Initial expressions derived from logic table (Z1, Z2):

A B C D | Z1 Z2

0 0 0 0 | 1  0

0 0 0 1 | 1  0

0 0 1 0 | 1  0

0 0 1 1 | 1  0

0 1 0 0 | 1  0

0 1 0 1 | 1  0

0 1 1 0 | 1  0

0 1 1 1 | 0  1

1 0 0 0 | 1  0

1 0 0 1 | 1  0

1 0 1 0 | 1  0

1 0 1 1 | 0  1

1 1 0 0 | 1  0

1 1 0 1 | 0  1

1 1 1 0 | 0  1

1 1 1 1 | 0  1

Z1 =

____   ___    __ _   __

ABCD + ABCD + ABCD + ABCD +

_ __   _ _    _  _    ___

ABCD + ABCD + ABCD + ABCD +

__     _ _     __

ABCD + ABCD + ABCD

Z2 =

_       _       _       _

ABCD + ABCD + ABCD + ABCD + ABCD

From the logic table and initial expressions, observe that Z1 is the complement of Z2, such that:

_           _

Z1=Z2 or Z1=Z2

Simplification of Z1 and Z2 via K-map:

Final expressions, assembled from K-map groupings:

__   __   __   __   __   __

Z1 =	AB + CD + AC + BC + AD + BD

Z2 = ABD + BCD + ACD + ABC

Z1	       _    __   _

AB | AB | AB | AB

CD	|    |    |  • |    |

_ 	|----|----|----|----|

CD	|    |  • |  • |  • |

__	|----|----|----|----|

CD	|  • |  • |  • |  • |

_	|----|----|----|----|

CD	|    |  • |  • |  • |

Z2	       _    __   _

AB | AB | AB | AB

CD	|  • |  • |    |  • |

_ 	|----|----|----|----|

CD	|  • |    |    |    |

__	|----|----|----|----|

CD	|    |    |    |    |

_	|----|----|----|----|

CD	|  • |    |    |    |

Using truth table, verification could be performed, showing that:

_           _

Z1=Z2 or Z1=Z2

but the table contains at least 19 columns of 1's and 0's, and is left for students to evaluate for practise.